Integration Note for the Hardware Team

How to reach the server and what to send it

| Project | RFID + Fingerprint Attendance and Access Control |
| --- | --- |
| Sprint / Story | Sprint 1 — US_2003 (web development side) |
| Author | Akshit Saini |
| Date | 1 September 2026 |
| Version | 0.1 — draft |

# 1. Where the server is

While we are developing, the server runs on my laptop and listens on port 3000:

http://<my-laptop-lan-ip>:3000

I will post the current IP in the group before each session — it changes when the router hands out a new lease. Both your boards and my laptop must be on the same Wi-Fi.

Important: localhost on the ESP32 means the ESP32 itself. It must be my IP address. If you cannot reach me, my firewall may be blocking port 3000 — tell me and I will open it.

# 2. Your device keys

| Board | device_key | Door |
| --- | --- | --- |
| Board 1 | DEV-MAIN-8F2A41C7 | Main Entrance |
| Board 2 | DEV-LAB-5C9E30B4 | Hardware Lab (restricted — card and finger) |

Send it with every request. An unknown key gets a 401.

# 3. What to send on a scan

POST /api/scan
{
  "device_key": "DEV-MAIN-8F2A41C7",
  "card_uid": "04A2B7C1",
  "fingerprint_slot": 1,
  "scanned_at": "2026-09-01T09:30:44Z",
  "device_event_id": "MAIN-0011"
}

- card_uid: uppercase hex, no colons or spaces.

- fingerprint_slot: the R307S template ID that matched, or null if no finger was read.

- device_event_id: unique per board, and it must survive a reboot. This is what stops a duplicate when you retry.

You get back:

{ "allowed": true, "direction": "IN", "person_name": "Akshit Saini", "reason": "OK" }

Fire the relay only when allowed is true. The reason string is there for the LED and buzzer — FINGERPRINT_REQUIRED should prompt for the finger rather than just refusing.

# 4. When Wi-Fi is down

Buffer locally, then send up to 50 at a time to POST /api/sync with the same fields. Re-sending the same batch is safe: anything already stored comes back as a duplicate and is not counted twice.

# 5. Heartbeat

POST /api/heartbeat every 60 seconds with device_key and firmware_ver. The reply carries server_time, which you can use to correct clock drift.

# 6. Test it in two minutes

curl -X POST http://<my-ip>:3000/api/scan \
  -H "Content-Type: application/json" \
  -d '{"device_key":"DEV-MAIN-8F2A41C7","card_uid":"04A2B7C1","fingerprint_slot":1,"scanned_at":"2026-09-01T09:30:44Z","device_event_id":"TEST-0001"}'

Arduino side, the same call:

HTTPClient http;
http.begin("http://192.168.1.24:3000/api/scan");
http.addHeader("Content-Type", "application/json");
String body = "{\"device_key\":\"DEV-MAIN-8F2A41C7\",\"card_uid\":\"" + uid +
              "\",\"fingerprint_slot\":" + String(slot) +
              ",\"scanned_at\":\"" + isoTime + "\",\"device_event_id\":\"" + eventId + "\"}";
int code = http.POST(body);
String reply = http.getString();
http.end();

Use a fresh device_event_id each time, or you will get duplicate back — which is the system working correctly.

# 7. Things I need from you

- Confirm the event counter survives a reboot.

- Tell me whether you will set the clock from NTP or rely on my server_time.

- Tell me how many events the offline buffer can hold.

- Send me the real card UIDs once you can read them, so I can register them against people.